# Warehouse Robotics – Path Optimization (Wokwi Simulation)

# AIM:

To simulate and implement a Warehouse Robot Path Optimization system using Arduino UNO, A4988 stepper motor driver, Bipolar stepper motor, HC-SR04 Ultrasonic sensor, and PIR Motion sensor in Wokwi software.

# COMPONENTS REQUIRED:

1. Arduino UNO

2. A4988 Stepper Motor Driver

3. Bipolar Stepper Motor

4. HC-SR04 Ultrasonic Sensor

5. PIR Motion Sensor

6. Jumper Wires and Power Connections (in Wokwi)

# CIRCUIT CONNECTIONS:

1️⃣ A4988 Stepper Motor Driver → Arduino UNO

| A4988 Pin | Arduino Pin |
| --- | --- |
| STEP | D3 |
| DIR | D4 |
| ENABLE | GND |
| VMOT | 12V |
| GND (Power) | GND |
| 1A, 1B, 2A, 2B | Stepper Motor Coils |

2️⃣ HC-SR04 Ultrasonic Sensor → Arduino UNO

| Ultrasonic Pin | Arduino Pin |
| --- | --- |
| VCC | 5V |
| GND | GND |
| TRIG | D8 |
| ECHO | D9 |

3️⃣ PIR Motion Sensor → Arduino UNO

| PIR Pin | Arduino Pin |
| --- | --- |
| VCC | 5V |
| OUT | D7 |
| GND | GND |

# PROGRAM:

#define STEP_PIN 3
#define DIR_PIN 4
#define TRIG_PIN 8
#define ECHO_PIN 9
#define PIR_PIN 7

void setup() {
  pinMode(STEP_PIN, OUTPUT);
  pinMode(DIR_PIN, OUTPUT);
  pinMode(TRIG_PIN, OUTPUT);
  pinMode(ECHO_PIN, INPUT);
  pinMode(PIR_PIN, INPUT);
  Serial.begin(9600);
}

long readDistance() {
  digitalWrite(TRIG_PIN, LOW);
  delayMicroseconds(2);
  digitalWrite(TRIG_PIN, HIGH);
  delayMicroseconds(10);
  digitalWrite(TRIG_PIN, LOW);
  long duration = pulseIn(ECHO_PIN, HIGH);
  return duration * 0.034 / 2;  // convert to cm
}

void stepMotor(int steps, bool dir) {
  digitalWrite(DIR_PIN, dir);
  for (int i = 0; i < steps; i++) {
    digitalWrite(STEP_PIN, HIGH);
    delayMicroseconds(800);
    digitalWrite(STEP_PIN, LOW);
    delayMicroseconds(800);
  }
}

void loop() {
  long distance = readDistance();
  int motion = digitalRead(PIR_PIN);

  if (motion == HIGH) {
    Serial.println("Motion detected! Stopping...");
    delay(1000);
  } else if (distance < 10) {
    Serial.println("Obstacle detected! Reversing...");
    stepMotor(200, false); // reverse
    delay(500);
  } else {
    Serial.println("Path clear! Moving forward...");
    stepMotor(200, true);  // forward
  }

  delay(100);
}

# OUTPUT:

The Serial Monitor displays messages indicating the robot’s path decision:

1. Path clear! Moving forward... → Robot moves forward.

2. Obstacle detected! Reversing... → Robot reverses its direction.

3. Motion detected! Stopping... → Robot pauses temporarily.

# RESULT:

The Warehouse Robotics Path Optimization system was successfully simulated using Wokwi. The robot detects obstacles and human motion, then optimizes its path by moving forward, stopping, or reversing based on sensor input.